Dr. A. Q. khan Institute of Computer Sciences and

Information Technology Kahuta

Department of Computer Science

Assignment No-1

Submitted By:

Muhammad Umair

Registration No. :

232202013

Class :

BSCS-6

Subject :

Computer Network Lab

Submitted To :

Mam Nayab

1. Unshielded Twisted Pair (UTP) Cable:

Unshielded Twisted Pair (UTP) cable is the most widely used copper cabling type for local area networks (LANs). It consists of pairs of insulated copper wires twisted together without any metallic shielding, designed to reduce electromagnetic interference (EMI) and crosstalk.

### Structure:

A UTP cable contains four pairs (eight individual wires) of copper conductors. Each pair is color-coded and twisted to reduce crosstalk between them. The cable also has a plastic outer jacket for protection.

| Color Pair | Wire Colors |
| --- | --- |
| Pair 1 | Blue & White/Blue |
| Pair 2 | Orange & White/Orange |
| Pair 3 | Green & White/Green |
| Pair 4 | Brown & White/Brown |

Working principle:

Signals are transmitted through copper wires as electrical pulses. Twisting each pair ensures that any external interference affects both wires equally, which can then be canceled out at the receiver’s end.

### Categories:

| Category | Max Speed | Bandwidth | Use |
| --- | --- | --- | --- |
| Cat 3 | 10 Mbps | 16 MHz | Telephone lines |
| Cat 5 | 100 Mbps | 100 MHz | Ethernet LANs |
| Cat 5e | 1 Gbps | 100 MHz | Fast Ethernet |
| Cat 6 | 10 Gbps | 250 MHz | Gigabit Ethernet |
| Cat 6a | 10 Gbps | 500 MHz | High-speed networks |
| Cat 7 | 10 Gbps | 600 MHz | Data centers |

### Advantages:

• Low cost and easy to install
• Lightweight and flexible
• Commonly available
• Suitable for short-distance communication

### Disadvantages:

• Susceptible to interference
• Shorter distance compared to fiber optic
• Not suitable for very high frequencies

### Applications:

• Ethernet LAN connections
• Telephone networks
• Home and office cabling

2. Shielded Twisted Pair (STP) Cable:

Shielded Twisted Pair (STP) cable is similar to UTP but includes an extra metallic shield around each twisted pair or around all pairs. This shielding reduces electromagnetic interference and provides more reliable data transmission in noisy environments.

### Structure:

STP cables contain four twisted pairs of copper wires. Each pair or all pairs are covered by a foil or braided shield and grounded to prevent electrical noise.

Types of Shielding:
1. FTP (Foiled Twisted Pair): Foil around all pairs.
2. STP (Shielded Twisted Pair): Braided shield around all pairs.
3. S/FTP: Foil around each pair and braided shield overall.

### Types of STP Cable Connections:

A) Straight-Through Cable — Straight-through cables are used to connect different devices, such as a computer to a switch or router.

- Both ends use the same wiring standard (either T568A or T568B).

- Pin 1 connects to Pin 1, Pin 2 to Pin 2, and so on.

Usage Examples:

- PC → Switch

- Router → Modem

B) Cross-Over Cable — Crossover cables are used to connect similar devices, such as two computers or two switches directly.

- One end uses T568A, the other end uses T568B.

- The Transmit (Tx) and Receive (Rx) wires are swapped.

Usage Examples:

- PC ↔ PC

- Switch ↔ Switch

- Router ↔ Router

C) Rollover Cable (Console Cable) — Rollover cables, also called Cisco console cables, connect a computer terminal to the console port of a router or switch for configuration.

- The pin numbers are completely reversed — pin 1 connects to pin 8, pin 2 to pin 7, etc.

- Usually flat and light-blue in color.

Usage Example:

- PC (COM port) ↔ Cisco Router/Switch (Console port)

Advantages:

• High protection against interference
• Better signal quality than UTP
• Suitable for industrial environments

### Disadvantages:

• More expensive and heavier
• Difficult to install
• Requires proper grounding

### Applications:

• Data centers
• Industrial LANs
• Environments with heavy machinery

3. Coaxial Cable:

A coaxial cable (coax) is a type of electrical cable with a central conductor, insulating layer, metallic shield, and outer protective jacket. The design prevents signal loss and interference, making it ideal for TV and broadband use.

### Structure:

1. Inner Conductor — Carries the signal.
2. Dielectric Insulator — Separates the conductor from the shield.
3. Outer Shield — Protects against EMI.
4. Outer Jacket — Provides protection.

Types of Coaxial Cables:

| Type | Impedance | Common Use |
| --- | --- | --- |
| RG-6 | 75 Ω | Cable TV, Internet |
| RG-59 | 75 Ω | CCTV, Analog video |
| RG-58 | 50 Ω | Ethernet (10Base2) |
| RG-11 | 75 Ω | Long distance TV signals |

### Advantages:

• Better shielding and noise protection
• Supports higher frequencies
• Long-distance capability

### Disadvantages:

• Bulky and less flexible
• Expensive connectors
• Difficult installation

### Applications:

• Cable television distribution
• CCTV systems
• Internet connections
• Early Ethernet networks

4. Fiber Optic Cable:

Fiber optic cable transmits data as light pulses through glass or plastic fibers. It offers extremely high bandwidth and speed, making it ideal for long-distance communication.

### Structure:

1. Core — Central glass fiber carrying light.
2. Cladding — Reflects light back into the core.
3. Buffer Coating — Protects from damage.
4. Outer Jacket — Provides outer protection.

Types of Fiber Optic Cable:

| Type | Core Diameter | Light Source | Distance | Use |
| --- | --- | --- | --- | --- |
| Single-Mode Fiber (SMF) | 8–10 µm | Laser | Up to 100 km | Long-distance communication |
| Multi-Mode Fiber (MMF) | 50–62.5 µm | LED | Up to 2 km | Local networks |

### Advantages:

• Very high data transmission rate
• Immune to EMI and RFI
• Long distance and secure transmission
• Lightweight and durable

### Disadvantages:

• Expensive to install and maintain
• Fragile and requires special connectors
• Difficult to splice or repair

### Applications:

• Internet backbone
• Long-distance telecom systems
• Data centers
• Medical and industrial equipment

5. Color Coding — T568A and T568B Wiring Standards:

The T568A and T568B are two wiring standards defined by TIA/EIA-568 for terminating UTP/STP cables in RJ45 connectors. Each cable has eight wires (four pairs). The color codes determine how each wire is connected to the pins.

### T568A Wiring Standard:

| Pin | Color | Function |
| --- | --- | --- |
| 1 | White/Green | Transmit + (Tx+) |
| 2 | Green | Transmit - (Tx-) |
| 3 | White/Orange | Receive + (Rx+) |
| 4 | Blue | Not used |
| 5 | White/Blue | Not used |
| 6 | Orange | Receive - (Rx-) |
| 7 | White/Brown | Not used |
| 8 | Brown | Not used |

### T568B Wiring Standard:

| Pin | Color | Function |
| --- | --- | --- |
| 1 | White/Orange | Transmit + (Tx+) |
| 2 | Orange | Transmit - (Tx-) |
| 3 | White/Green | Receive + (Rx+) |
| 4 | Blue | Not used |
| 5 | White/Blue | Not used |
| 6 | Green | Receive - (Rx-) |
| 7 | White/Brown | Not used |
| 8 | Brown | Not used |

### Difference Between T568A and T568B:

| Feature | T568A | T568B |
| --- | --- | --- |
| Order of wire pairs | Green first | Orange first |
| Common usage | Residential wiring | Commercial/Ethernet |
| Compatibility | Used in government networks | Used in private industries |
| Performance | Same | Same |

## Comparison of All Cable Types:

| Cable Type | Medium | Speed | Distance | Interference Resistance | Cost | Typical Use |
| --- | --- | --- | --- | --- | --- | --- |
| UTP | Copper | Up to 10 Gbps | 100 m | Low | Low | LAN, phones |
| STP | Copper | Up to 10 Gbps | 100 m | High | Medium | Industrial LAN |
| Coaxial | Copper | Up to 1 Gbps | 500 m | High | Medium | TV, CCTV |
| Fiber Optic | Glass/Plastic | Up to 100 Tbps | 100 km+ | Excellent | High | Backbone, WAN |

## Conclusion:

Each cable type serves a unique purpose. UTP and STP are widely used in LANs, Coaxial is common in TV and CCTV, and Fiber Optic dominates modern long-distance communication. Understanding T568A and T568B standards ensures correct cabling and reliable network performance.

**********************************